def hamming_encode(data):

m = len(data)

r = 0

# Find number of parity bits

while (2 ** r) < (m + r + 1):

r += 1

n = m + r

code = [0] * (n + 1)

# Put data bits in positions other than 1,2,4,8...

j = 0

for i in range(1, n + 1):

if (i & (i - 1)) != 0:

code[i] = int(data[j])

j += 1

# Calculate parity bits

for p in range(r):

pos = 2 ** p

parity = 0

for i in range(1, n + 1):

if i & pos:

parity ^= code[i]

code[pos] = parity

# Display from highest position to lowest

codeword = ""

for i in range(n, 0, -1):

codeword += str(code[i])

return codeword, r

def check_error(received, r):

n = len(received)

# Store bits according to Hamming positions

code = [0] * (n + 1)

j = 1

for i in range(n - 1, -1, -1):

code[j] = int(received[i])

j += 1

error_position = 0

# Calculate syndrome

for p in range(r):

pos = 2 ** p

parity = 0

for i in range(1, n + 1):

if i & pos:

parity ^= code[i]

if parity:

error_position += pos

return error_position, code

# ---------------- MAIN PROGRAM ----------------

data = input("Enter data bits: ")

codeword, r = hamming_encode(data)

print("\nNumber of parity bits:", r)

print("Transmitter codeword:", codeword)

received = input("\nEnter received codeword: ")

error_position, code = check_error(received, r)

if error_position == 0:

print("No error detected.")

else:

print("Error detected at bit position:", error_position)

# Correct the error

code[error_position] ^= 1

corrected = ""

for i in range(len(received) - 1, -1, -1):

corrected += str(code[len(received) - i])

print("Corrected codeword:", corrected)

# Extract original data

data_received = ""

for i in range(1, len(received) + 1):

if (i & (i - 1)) != 0:

data_received += str(code[i])

print("Recovered data:", data_received)

Assignment 2 Code

# Display from highest position to lowest

codeword = ""

for i in range(n, 0, -1):

codeword += str(code[i])

return codeword, r

def check_error(received, r):

n = len(received)

# Store bits according to Hamming positions

code = [0] * (n + 1)

j = 1

for i in range(n - 1, -1, -1):

code[j] = int(received[i])

j += 1

error_position = 0

# Calculate syndrome

for p in range(r):

pos = 2 ** p

parity = 0

for i in range(1, n + 1):

if i & pos:

parity ^= code[i]

if parity:

error_position += pos

return error_position, code

# ---------------- MAIN PROGRAM ----------------

data = input("Enter data bits: ")

codeword, r = hamming_encode(data)

print("\nNumber of parity bits:", r)

print("Transmitter codeword:", codeword)

received = input("\nEnter received codeword: ")

error_position, code = check_error(received, r)

if error_position == 0:

print("No error detected.")

else:

print("Error detected at bit position:", error_position)

# Correct the error

code[error_position] ^= 1

corrected = ""

for i in range(len(received) - 1, -1, -1):

corrected += str(code[len(received) - i])

print("Corrected codeword:", corrected)

# Extract original data

data_received = ""

for i in range(1, len(received) + 1):

if (i & (i - 1)) != 0:

data_received += str(code[i])

print("Recovered data:", data_received)

# ---------------- MAIN PROGRAM ----------------

data = input("Enter data bits: ")

codeword, r = hamming_encode(data)

print("\nNumber of parity bits:", r)

print("Transmitter codeword:", codeword)

received = input("\nEnter received codeword: ")

error_position, code = check_error(received, r)

if error_position == 0:

print("No error detected.")

else:

print("Error detected at bit position:", error_position)

# Correct the error

code[error_position] ^= 1

corrected = ""

for i in range(len(received) - 1, -1, -1):

corrected += str(code[len(received) - i])

print("Corrected codeword:", corrected)

# Extract original data

data_received = ""

for i in range(1, len(received) + 1):

if (i & (i - 1)) != 0:

data_received += str(code[i])

print("Recovered data:", data_received)